# Assignment 4: Exploring Instruction-Level Parallelism (ILP) in Modern Processors

Student: Vamsi Krishna
Course: (fill in)
Date: February 1, 2026

# Part 1 — Critical Review and Synthesis (Contemporary Research)

Note: This report is written to be copy/paste reproducible. Replace the example numbers and generated screenshots with your own after running gem5 on your Mac.

## 1. Evolution of ILP

Instruction-Level Parallelism (ILP) is the ability to execute multiple machine instructions concurrently. Key milestones include early pipelining, dynamic scheduling (e.g., Tomasulo’s algorithm), superscalar multiple-issue designs, aggressive branch prediction and speculative execution, and large out-of-order (OoO) instruction windows. Classic work quantified limits of ILP and highlighted the dominant role of control flow and memory dependencies in constraining achievable IPC.

Selected historical milestones (brief):

1960s: Dynamic scheduling with register renaming concepts (IBM System/360 Model 91; Tomasulo algorithm).

1980s–1990s: Superscalar OoO becomes mainstream; detailed studies of ILP limits (Wall, 1991).

1990s–2000s: Deeper pipelines + better predictors; EPIC/VLIW explored but general-purpose adoption limited.

2010s–2020s: Diminishing returns from wider OoO; energy constraints; shift toward heterogeneous and accelerator-rich systems.

## 2. Core ILP Concepts

### 2.1 Parallelism Detection and Exploitation

Modern processors detect ILP using a combination of static (compiler) and dynamic (hardware) techniques. OoO cores create a window of in-flight instructions; register renaming removes false dependencies; issue queues and wakeup/select logic identify ready instructions; speculation (branch prediction, memory dependence prediction) allows execution beyond unresolved control and memory operations.

### 2.2 Fundamental ILP Limitations

ILP is fundamentally limited by: (1) true data dependencies (RAW), especially long-latency operations; (2) control dependencies due to branches (misprediction causes squashes and lost work); (3) memory ordering/aliasing uncertainty (loads may depend on earlier stores); and (4) finite resources (rename registers, issue bandwidth, ROB size, functional unit availability).

### 2.3 Performance Metrics and Trade-offs

Common ILP effectiveness metrics include IPC/throughput, CPI, average latency per instruction, branch misprediction rate, and memory-level parallelism (MLP). Increasing ILP capability typically raises hardware complexity and power: wide issue, large instruction windows, and sophisticated predictors improve throughput but increase area/energy and may deliver diminishing marginal gains.

## 3. Contemporary Research (Last 5 years) — Critical Review

Papers selected from reputable IEEE/ACM venues (ISCA, HPCA, ISPASS, ACM DL).

Chadwick et al., “The Future of Instruction-Level Parallelism (ILP),” IEEE ISPASS 2025. Revisits theoretical ILP limits using dynamic dependency graphs; argues modern cores are approaching ILP bounds.

Kim & Ros, “Effective Context-Sensitive Memory Dependence Prediction,” IEEE HPCA 2024. Improves memory dependence prediction to unlock more safe speculation for loads, increasing ILP/MLP.

Bera et al., “Constable: Improving Performance and Power Efficiency by Safely Eliminating Load Instruction Execution,” 2024. Proposes eliminating certain load executions safely to reduce pipeline pressure and energy while retaining correctness.

Chaturvedi et al., “GhOST: A GPU Out-of-Order Scheduling Technique for Stall Reduction,” ACM/IEEE ISCA 2024. Shows how OoO-like scheduling ideas can be adapted to GPUs to reduce stalls and expose more instruction-level overlap.

Xu et al., “Mosaic: Exploiting Instruction-Level Parallelism on Deep Learning Accelerators,” ACM 2025. Compiler-level tessellation approach to expose ILP in DNN accelerators, emphasizing cross-layer scheduling.

### 3.1 Cross-paper Synthesis: What’s changing and why

Across these works, a recurring theme is that classic ILP techniques (wider OoO, deeper speculation) are nearing practical limits due to power/complexity. Research is therefore shifting to: (a) more precise speculation (especially memory dependence prediction), (b) reducing wasted work (e.g., avoiding redundant load execution), and (c) pushing ILP-like ideas into accelerators and heterogeneous systems where the cost/performance envelope differs.

### 3.2 Key Challenges Identified

Diminishing returns: Wider issue/ROB grows power disproportionately, and ILP headroom is small in many workloads.

Speculation cost: Mispredictions and misspeculation waste energy and reduce effective throughput.

Memory wall: Long-latency loads and uncertain aliasing still dominate stalls despite large windows and prefetching.

Verification and security: Complex speculation increases validation burden and interacts with side-channel risk.

### 3.3 How researchers address them

Smarter dependence prediction to safely execute loads earlier (context-sensitive predictors).

Microarchitectural techniques to avoid executing some loads when their values can be inferred/reused safely.

Accelerator-centric scheduling and compiler transformations to restructure instruction streams for higher overlap.

Reconfigurable / adaptive microarchitecture knobs (queue sizes, policies) to match workload phases.

## 4. Future Directions

Most promising directions suggested by the literature and broader trends include:
• Hybrid ILP+TLP: Using SMT, chiplets, and heterogeneous cores to trade ILP complexity for parallel threads.
• Specialization: Domain-specific accelerators where ILP can be exposed at compile time with lower control-flow uncertainty.
• ML-guided microarchitecture: Predictors and schedulers that learn per-workload behavior, constrained by energy and verifiability.
• Security-aware speculation: Techniques that preserve performance while limiting speculative side effects.

# Part 2 — Practical Exploration in gem5

## 0. Environment and Repository Layout

gem5 repo path example: vamsikrishna@macbookpro gem5 %
This submission includes: configuration scripts (assignment4_ilp/configs), benchmarks (assignment4_ilp/benchmarks), helper scripts (assignment4_ilp/scripts), and example outputs/figures.

## 1. Build and Sanity Check (Hello World)

Example terminal transcript (generated; replace with your own screenshot):

## 2. Basic Pipeline Simulation (MinorCPU)

MinorCPU is an in-order core with a fixed pipeline and supports MinorTrace/minorview visualization. We run hello and an ILP microbenchmark and inspect per-cycle instruction movement.

Pipeline visualization example (generated):

## 3. Performance Metrics (Throughput & Latency)

We use gem5 stats (stats.txt) to compute IPC (inst/cycle) and CPI (cycles/inst). The included script scripts/parse_stats.py extracts sim_insts and cycle count to compute metrics.

## 4. Impact of Branch Prediction

Using O3CPU, we compare a weak predictor (StaticBP) against a stronger predictor (LTAGE). More accurate prediction reduces pipeline squashes, improving IPC.

## 5. Multiple Issue (Superscalar) Exploration

We vary the O3 width parameters (fetch/decode/issue/commit) from width=1 to width=4. Benchmarks: integer mix (ilp_int), floating point (ilp_fp), and memory streaming (ilp_mem). Superscalar shines most on instruction mixes with sufficient independent operations and few hard dependencies.

## 6. SMT (Simultaneous Multithreading)

We enable SMT with numThreads=2 on a single O3 core, running two independent SE workloads concurrently. We monitor throughput and identify contention: shared fetch/issue bandwidth, ROB pressure, cache bandwidth.

## 7. Example Charts

Example plots generated from sample data (replace with your measured results):

## 8. Key Questions — Discussion

Interactions: Strong branch prediction increases the value of wide issue and large windows; memory dependence prediction increases both ILP and MLP.

Limitations: True dependencies and cache misses cap gains; wider designs increase contention under SMT.

Balance: Use profiling-driven widths and predictor complexity; consider heterogeneous cores + accelerators instead of ever-wider OoO.

## 9. GitHub Repository (Template)

I cannot create or host a GitHub repository from here. Create one locally and push this folder.
Suggested repo name: cs-xxx-assignment4-ilp
Template URL: https://github.com/<your-username>/cs-xxx-assignment4-ilp

# References (APA)

Anderson, D. W., Sparacio, F. J., & Tomasulo, R. M. (1967). The IBM System/360 Model 91: Machine philosophy and instruction-handling. IBM Journal of Research and Development.

Chadwick, A. W., et al. (2025). The Future of Instruction-Level Parallelism (ILP). IEEE International Symposium on Performance Analysis of Systems and Software (ISPASS).

Chaturvedi, I., et al. (2024). GhOST: A GPU Out-of-Order Scheduling Technique for Stall Reduction. ACM/IEEE International Symposium on Computer Architecture (ISCA).

Kim, S. S., & Ros, A. (2024). Effective Context-Sensitive Memory Dependence Prediction. IEEE International Symposium on High-Performance Computer Architecture (HPCA).

Rau, B. R., & Fisher, J. A. (1993). Instruction-level parallel processing: History, overview, and perspective. Journal of Supercomputing.

Tomasulo, R. M. (1967). An Efficient Algorithm for Exploiting Multiple Arithmetic Units. IBM Journal of Research and Development.

Wall, D. W. (1991). Limits of instruction-level parallelism. ACM/IEEE Architectural Support for Programming Languages and Operating Systems / or equivalent archival listing.

Xu, J., et al. (2025). Mosaic: Exploiting Instruction-Level Parallelism on Deep Learning Accelerators. ACM.

gem5 documentation (2025). Minor CPU model, O3 pipeline visualization, and gem5 statistics guides.